# Lab 02D

Part 1) Ubuntu Server 18.04.1 Server

1. In VMplayer, add a new hard disk of size 20 GB.

2. From the command prompt, display all disks.

3. Create three primary partitions as specified below:

Size      File System       Mount Point

10 GB         ext2             /media/diskc1

6 GB         ext3            /media/diskc2

4 GB         ext4            /media/diskc3

4. From the command prompt, display all partitions.

Extra Credit:

Mount all disks at boot time; show your work.

Part 2) Fedora Server 28-1.1

1. In VMplayer, add a new hard disk of size 20 GB.

2. From the command prompt, display all disks.

3. Create three primary partitions as specified below:

Size      File System       Mount Point

10 GB         ext2             /media/diskc1

6 GB         ext3            /media/diskc2

4 GB         ext4            /media/diskc3

4. From the command prompt, display all partitions.

Extra Credit:

Mount all disks at boot time; show your work.